Communication between esp 32 and Rasberry PI , UART Connection

Great — you already have ESP32 code uploading and the Pi terminal open. Now I’ll give you the exact step-by-step sequence from zero to final proof, with the exact commands + which code to run first, and what output you should see.

This is ONLY for UART wired communication test (no app/UI).

0) One-time hardware check (do first)

0.1 Wiring (must be EXACT)

Raspberry Pi 4B (40-pin header):

- Pin 8 (GPIO14 / TXD0) → ESP32 RX = GPIO16

- Pin 10 (GPIO15 / RXD0) → ESP32 TX = GPIO17

- Pin 6 (GND) → ESP32 GND

✅ TX ↔ RX crossed
✅ Common GND
✅ Do NOT connect Pi 5V to ESP32

Very common mistake: using the wrong physical pin numbers because the header is flipped. Pin-1 is the 3.3V corner; from there pins count 1–2, 3–4… down.

1) Raspberry Pi: enable UART correctly (one time)

1.1 raspi-config

On Pi terminal:

sudo raspi-config

Go:

- Interface Options → Serial Port

  - Login shell over serial? → NO

  - Enable serial hardware? → YES
Reboot.

1.2 Disable Bluetooth UART (Pi 4 IMPORTANT)

Bluetooth often steals the “good” UART. Disable it:

sudo systemctl disable --now hciuart

Edit config:

sudo nano /boot/config.txt

Add at bottom:

enable_uart=1
dtoverlay=disable-bt

Reboot:

sudo reboot

1.3 Confirm which device /dev/serial0 really is

After reboot:

ls -l /dev/serial0
readlink -f /dev/serial0

✅ You should see it point to something like /dev/ttyAMA0 or /dev/ttyS0.

2) FIRST test: Raspberry Pi UART by itself (loopback) ✅

This proves the Pi UART works before involving ESP32.

2.1 Temporarily short Pi TX to Pi RX

- Connect Pi pin 8 to Pi pin 10 using one jumper (temporary).

2.2 Set the port speed and open a reader

Run:

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo
sudo cat /dev/serial0

Now in another terminal (or stop cat with Ctrl+C and run the send command):

printf "PI_LOOPBACK_TEST\n" | sudo tee /dev/serial0

✅ If loopback is OK, you will see:
PI_LOOPBACK_TEST

Then remove the short wire and reconnect Pi to ESP32.

3) SECOND test: ESP32 → Pi (one-way) ✅

This proves ESP32 TX is reaching Pi RX.

3.1 Upload this ESP32 code FIRST (Arduino IDE)

(Your sending code is correct; use this exactly)

#include <HardwareSerial.h>
HardwareSerial PiSerial(2);

void setup() {
  Serial.begin(115200);
  PiSerial.begin(115200, SERIAL_8N1, 16, 17); // RX=16 TX=17
  Serial.println("ESP32 sending to Pi...");
}

void loop() {
  PiSerial.println("HELLO_FROM_ESP32");
  delay(1000);
}

3.2 On Pi terminal: set UART config + read

Run:

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo
sudo cat /dev/serial0

✅ You must see:

HELLO_FROM_ESP32
HELLO_FROM_ESP32
...

If you see nothing here

It is almost always:

- TX/RX pins swapped or wrong physical pins

- missing Pi↔ESP32 GND

- /dev/serial0 not pointing to the UART you enabled

- baud not set (that’s why we used stty)

4) THIRD test: Pi → ESP32 (one-way) ✅

This proves Pi TX is reaching ESP32 RX.

4.1 Upload this ESP32 receiver code

(This version treats both \r and \n as end-line, so no blank lines.)

#include <HardwareSerial.h>
HardwareSerial PiSerial(2);

String buf = "";

void setup() {
  Serial.begin(115200);
  PiSerial.begin(115200, SERIAL_8N1, 16, 17);
  Serial.println("ESP32 ready. Waiting for Pi...");
}

void loop() {
  while (PiSerial.available()) {
    char c = (char)PiSerial.read();
    if (c == '\r' || c == '\n') {
      if (buf.length() > 0) {
        Serial.print("From Pi: ");
        Serial.println(buf);
        buf = "";
      }
    } else {
      if (buf.length() < 200) buf += c;
    }
  }
}

4.2 On Pi terminal: send a line

Run:

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo
printf "TEST_FROM_PI\n" | sudo tee /dev/serial0

✅ ESP32 Serial Monitor should show:
From Pi: TEST_FROM_PI

Earlier you saw From Pi: with blank — that happens when only \r is received or an empty line is sent. The code above fixes that.

5) FINAL test: Two-way (Pi sends, ESP32 replies ACK) ✅✅

5.1 Upload this ESP32 echo code

#include <HardwareSerial.h>
HardwareSerial PiSerial(2);

String buf = "";

void setup() {
  Serial.begin(115200);
  PiSerial.begin(115200, SERIAL_8N1, 16, 17);
  Serial.println("ESP32 echo mode ready...");
}

void loop() {
  while (PiSerial.available()) {
    char c = (char)PiSerial.read();
    if (c == '\r' || c == '\n') {
      if (buf.length() > 0) {
        Serial.print("From Pi: ");
        Serial.println(buf);

        PiSerial.print("ESP32_ACK: ");
        PiSerial.println(buf);

        buf = "";
      }
    } else {
      if (buf.length() < 200) buf += c;
    }
  }
}

5.2 On Pi terminal #1 (listen)

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo
sudo cat /dev/serial0

5.3 On Pi terminal #2 (send)

printf "PING\n" | sudo tee /dev/serial0

✅ Results:

- ESP32 monitor: From Pi: PING

- Pi terminal: ESP32_ACK: PING

That’s full proof communication works both directions.

Quick fix for what your photos show (no output on cat)

In your Pi terminal, before cat, always run this once:

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo

Then:

sudo cat /dev/serial0

This sets the correct baud + raw mode. Without it, Pi may stay at a different baud and you may see nothing/garbage.

If you want me to verify instantly

Run on Pi and paste output:

ls -l /dev/serial0
readlink -f /dev/serial0

And tell me: in Step 3, when ESP32 sends HELLO_FROM_ESP32, does cat /dev/serial0 show it after stty?

sudo systemctl disable --now hciuart after this it shows failed to disable unit : unit hciuart.serivce does not exist

That’s okay — it just means your Pi OS doesn’t have a service named hciuart (on some builds it’s named differently, or Bluetooth is handled another way). We can disable Bluetooth UART the correct way without that service.

1) Check what Bluetooth service exists

Run this on the Pi terminal:

systemctl list-unit-files | grep -i -E "bluetooth|hci|uart"

If you see bluetooth.service, that’s the one.

2) Disable Bluetooth (safe + common fix)

Run:

sudo systemctl disable --now bluetooth

(If it says not found, that’s also fine—your build may not be running Bluetooth at all.)

3) Disable Bluetooth UART via config.txt (works on Pi 4)

This is the most reliable method.

Open config:

sudo nano /boot/config.txt

Add these lines at the end:

enable_uart=1
dtoverlay=disable-bt

Save (Ctrl+O, Enter) → Exit (Ctrl+X)

Reboot:

sudo reboot

4) After reboot, verify UART device

ls -l /dev/serial0
readlink -f /dev/serial0

5) Also make sure serial login console is OFF (important)

Run:

sudo systemctl disable --now serial-getty@ttyAMA0.service 2>/dev/null
sudo systemctl disable --now serial-getty@ttyS0.service 2>/dev/null

Why this matters

If Bluetooth/console is still using the UART, you can get:

- random garbage characters

- empty From Pi: lines

- no output on /dev/serial0

If you paste the output of:

ls -l /dev/serial0
readlink -f /dev/serial0

I’ll tell you exactly which UART you’re using and confirm it’s correct.

Calibration code for the Temperature sensor

#include <Wire.h>

#include <DFRobot_MLX90614.h>

#define I2C_SDA 21

#define I2C_SCL 22

DFRobot_MLX90614_I2C mlx;

// -------------------- Practical Forehead Detection --------------------

// If room is warm, delta becomes smaller, so use dynamic threshold.

float deltaThreshold(float ambientC) {

// Cooler room -> need bigger delta, warmer room -> allow smaller delta

if (ambientC < 22.0) return 3.0;

if (ambientC < 26.0) return 2.5;

if (ambientC < 29.0) return 2.0;

return 1.5;

}

// Minimum plausible forehead surface temperature (not body core)

const float MIN_FOREHEAD_SURFACE_C = 28.0;   // allow lower than 30 because forehead surface can be 28–32

const float MAX_FOREHEAD_SURFACE_C = 42.0;

// -------------------- Calibration (optional) --------------------

const bool USE_LINEAR_CAL = false;   // start false; turn true after you compute A,B

const float CAL_A = 1.000;

const float CAL_B = 0.000;

const float OFFSET_C = 0.0;          // if you want simple offset instead

float applyCal(float t) {

if (USE_LINEAR_CAL) return CAL_A * t + CAL_B;

return t + OFFSET_C;

}

// -------------------- Emissivity --------------------

const float SKIN_EMISSIVITY = 0.95;

// -------------------- Sampling/Filtering (relaxed) --------------------

const int   SAMPLES = 20;

const int   SAMPLE_DELAY_MS = 40;   // ~0.8 sec batch

const float OUTLIER_BAND = 1.2;     // relaxed (was 0.6)

const float STABILITY_MAX_STD = 0.60; // relaxed (was 0.25)

const int   MIN_GOOD = 8;           // minimum good samples

// Warm-up (very important)

const int WARMUP_SEC = 10;          // for demo; in real use keep 2–5 minutes

// -------------------- Helpers --------------------

float meanOf(const float *arr, int n) {

float s = 0;

for (int i = 0; i < n; i++) s += arr[i];

return s / n;

}

float stdOf(const float *arr, int n, float mean) {

float s = 0;

for (int i = 0; i < n; i++) {

float d = arr[i] - mean;

s += d * d;

}

return sqrt(s / n);

}

float medianOf(float *arr, int n) {

for (int i = 0; i < n - 1; i++) {

for (int j = i + 1; j < n; j++) {

if (arr[j] < arr[i]) {

float t = arr[i]; arr[i] = arr[j]; arr[j] = t;

}

}

}

if (n % 2 == 1) return arr[n / 2];

return 0.5f * (arr[n / 2 - 1] + arr[n / 2]);

}

// Reads batch and returns stable averaged ambient/object

bool readBatch(float &ambAvg, float &objAvg, float &objStd, float &deltaC) {

float amb[SAMPLES];

float obj[SAMPLES];

for (int i = 0; i < SAMPLES; i++) {

amb[i] = mlx.getAmbientTempCelsius();

obj[i] = mlx.getObjectTempCelsius();

delay(SAMPLE_DELAY_MS);

}

// Robust median

float objCopy[SAMPLES];

for (int i = 0; i < SAMPLES; i++) objCopy[i] = obj[i];

float med = medianOf(objCopy, SAMPLES);

// Outlier rejection

float ambGood[SAMPLES];

float objGood[SAMPLES];

int good = 0;

for (int i = 0; i < SAMPLES; i++) {

if (fabs(obj[i] - med) <= OUTLIER_BAND) {

ambGood[good] = amb[i];

objGood[good] = obj[i];

good++;

}

}

if (good < MIN_GOOD) return false;

ambAvg = meanOf(ambGood, good);

objAvg = meanOf(objGood, good);

objStd = stdOf(objGood, good, objAvg);

deltaC = objAvg - ambAvg;

return true;

}

void setup() {

Serial.begin(115200);

delay(1200);

Serial.println("\n=== MLX90614 Forehead (Fixed Detection) ===");

Wire.begin(I2C_SDA, I2C_SCL);

Wire.setClock(100000); // SMBus safe

while (mlx.begin() != NO_ERR) {

Serial.println("❌ MLX90614 not detected. Check wiring (SDA21/SCL22/3.3V/GND).");

delay(1500);

}

mlx.setEmissivityCorrectionCoefficient(SKIN_EMISSIVITY);

Serial.print("✅ MLX ready. Emissivity = ");

Serial.println(SKIN_EMISSIVITY, 2);

Serial.print("Warm-up countdown (demo): ");

for (int i = WARMUP_SEC; i > 0; i--) {

Serial.print(i); Serial.print(" ");

delay(1000);

}

Serial.println("\nStart: aim at forehead (best distance 3–5 cm, straight angle).");

}

void loop() {

float amb, obj, objStd, delta;

bool ok = readBatch(amb, obj, objStd, delta);

if (!ok) {

Serial.println("⚠️ Batch unstable (too many outliers). Hold steady and try again.");

delay(400);

return;

}

float dTh = deltaThreshold(amb);

// Presence rule (more realistic):

// 1) Object must be above minimum forehead surface temp

// 2) Delta must exceed dynamic threshold

// 3) Noise (std) must not be too high

String reason = "";

bool present = true;

if (objStd > STABILITY_MAX_STD) { present = false; reason = "noisy/unstable"; }

else if (obj < MIN_FOREHEAD_SURFACE_C) { present = false; reason = "too cold (background)"; }

else if (delta < dTh) { present = false; reason = "delta too small"; }

// Apply calibration ONLY if present

float forehead = applyCal(obj);

// clamp sanity

if (present && (forehead < MIN_FOREHEAD_SURFACE_C || forehead > MAX_FOREHEAD_SURFACE_C)) {

present = false;

reason = "out of plausible range";

}

// Print debug always (so you can tune)

Serial.print("Ambient: "); Serial.print(amb, 2);

Serial.print(" | Object: "); Serial.print(obj, 2);

Serial.print(" | Δ: "); Serial.print(delta, 2);

Serial.print(" (th="); Serial.print(dTh, 1); Serial.print(")");

Serial.print(" | Std: "); Serial.print(objStd, 2);

if (!present) {

Serial.print("  => NO FOREHEAD (");

Serial.print(reason);

Serial.println(")");

} else {

Serial.print("  => Forehead: ");

Serial.print(forehead, 2);

Serial.println(" °C");

}

delay(500);

}

Code for sp02 and Respiration rate

#include <Wire.h>

#include "MAX30105.h"

#include "heartRate.h"

#include <math.h>

// ----------------- ESP32 I2C pins -----------------

#define SDA_PIN 21

#define SCL_PIN 22

// Some breakouts swap FIFO channels; if your RED/IR are swapped set to 1

#define SWAP_RED_IR 0

// Finger detect threshold (IR). Tune 20000~80000 depending on LED current & finger.

#define FINGER_ON_IR 50000

// ----------------- SpO2 window -----------------

#define N_SPO2 100   // ~1 second if sampleRate=100

// SpO2 calibration (optional):

// After comparing to a known pulse oximeter, you can map:

// spo2_cal = SPO2_A * spo2_raw + SPO2_B

const float SPO2_A = 1.000;

const float SPO2_B = 0.000;

// SpO2 smoothing

const float SPO2_ALPHA = 0.7;

// ----------------- BPM smoothing -----------------

const int BPM_AVG_N = 6;

float bpmBuf[BPM_AVG_N] = {0};

int bpmPtr = 0;

// Reject BPM outside realistic range for adults (adjust if needed)

const float BPM_MIN = 40.0;

const float BPM_MAX = 180.0;

// ----------------- Respiration (PPG-derived, experimental) -----------------

// Respiration from PPG amplitude modulation is approximate.

// Typical adult RR: 12–20 bpm (breaths/min)

const int RR_AVG_N = 5;

float rrBuf[RR_AVG_N] = {0};

int rrPtr = 0;

float rrValue = 0;

// Resp peak detection limits (ms)

const uint32_t RR_MIN_MS = 1500;   // 40 breaths/min

const uint32_t RR_MAX_MS = 10000;  // 6 breaths/min

// ----------------- Sensor -----------------

MAX30105 sensor;

// SpO2 buffers

uint32_t redBuf[N_SPO2];

uint32_t irBuf[N_SPO2];

int spo2Idx = 0;

float spo2Raw = 0;

float spo2Filt = 0;

// BPM

float bpmInstant = 0;

float bpmSmooth = 0;

uint32_t lastBeatMs = 0;

// Respiration processing state

// We estimate respiration from the slow envelope of (IR - baseline)

float irBaseline = 0;        // DC estimate

float envFast = 0;           // envelope (fast-ish)

float envSlow = 0;           // smoothed slow envelope (resp component)

float envMin = 1e9, envMax = 0;

uint32_t envWindowStart = 0;

float prev2 = 0, prev1 = 0, curr = 0;

uint32_t lastBreathMs = 0;

// Print timing

uint32_t lastPrintMs = 0;

// ----------------- Utils -----------------

static float mean_u32(const uint32_t *x, int n){

double s=0;

for(int i=0;i<n;i++) s += x[i];

return (float)(s/n);

}

static float rms_ac(const uint32_t *x, int n, float dc){

double s=0;

for(int i=0;i<n;i++){

double d = (double)x[i] - dc;

s += d*d;

}

return (float)sqrt(s/n);

}

// Ratio-of-ratios SpO2 (approx; requires calibration to be accurate)

static float calcSpO2(const uint32_t *red, const uint32_t *ir, int n){

float dcRed = mean_u32(red,n);

float dcIr  = mean_u32(ir,n);

if(dcRed<=0 || dcIr<=0) return -1;

float acRed = rms_ac(red,n,dcRed);

float acIr  = rms_ac(ir,n,dcIr);

if(acRed<=0 || acIr<=0) return -1;

float R = (acRed/dcRed)/(acIr/dcIr);

// common demo approximation

float s = 110.0f - 25.0f * R;

if(s>100) s=100;

if(s<70)  s=70;

return s;

}

static float pushAvg(float *buf, int n, int &ptr, float v){

buf[ptr] = v;

ptr = (ptr + 1) % n;

float sum = 0;

int cnt = 0;

for(int i=0;i<n;i++){

if(buf[i] > 0){

sum += buf[i];

cnt++;

}

}

return (cnt>0) ? (sum/cnt) : 0;

}

void resetAll() {

spo2Idx = 0;

spo2Raw = 0;

spo2Filt = 0;

bpmInstant = 0;

bpmSmooth = 0;

lastBeatMs = 0;

for(int i=0;i<BPM_AVG_N;i++) bpmBuf[i]=0;

bpmPtr = 0;

rrValue = 0;

for(int i=0;i<RR_AVG_N;i++) rrBuf[i]=0;

rrPtr = 0;

irBaseline = 0;

envFast = 0;

envSlow = 0;

envMin = 1e9;

envMax = 0;

envWindowStart = millis();

prev2 = prev1 = curr = 0;

lastBreathMs = 0;

}

void setup() {

Serial.begin(115200);

delay(1200);

Wire.begin(SDA_PIN, SCL_PIN);

Wire.setClock(100000); // stable start (you can try 400k later)

Serial.println("\n=== MAX30102 SpO2 + BPM + RR (FIFO Improved) ===");

if(!sensor.begin(Wire, I2C_SPEED_STANDARD)) {

Serial.println("❌ MAX30102 not found (I2C). Ensure scanner shows 0x57 and wiring is correct.");

while(1) delay(1000);

}

Serial.println("✅ MAX30102 detected");

// Stable sensor config

byte ledBrightness = 0x1F; // start moderate

byte sampleAverage = 4;

byte ledMode = 2;          // Red + IR

int sampleRate = 100;      // IMPORTANT: keep at 100 to match window assumptions

int pulseWidth = 411;

int adcRange = 4096;

sensor.setup(ledBrightness, sampleAverage, ledMode, sampleRate, pulseWidth, adcRange);

sensor.setPulseAmplitudeRed(0x1F);

sensor.setPulseAmplitudeIR(0x1F);

sensor.setPulseAmplitudeGreen(0);

// Optional: improve FIFO behavior

sensor.clearFIFO();

resetAll();

Serial.println("Place finger steady on sensor (no movement).");

}

void loop() {

// Pull new FIFO samples into library buffer

sensor.check();

while (sensor.available()) {

// Read FIFO sample

uint32_t red = sensor.getFIFORed();

uint32_t ir  = sensor.getFIFOIR();

#if SWAP_RED_IR

uint32_t tmp = red; red = ir; ir = tmp;

#endif

// Move to next

sensor.nextSample();

// Finger detection

if (ir < FINGER_ON_IR) {

// print occasionally even without finger

if (millis() - lastPrintMs >= 1000) {

lastPrintMs = millis();

Serial.println("No finger detected");

}

resetAll();

continue;

}

// ---------------- BPM (beat detection on IR) ----------------

if (checkForBeat(ir)) {

uint32_t now = millis();

if (lastBeatMs != 0) {

uint32_t dt = now - lastBeatMs;

// sanity range: 40..180 bpm  => dt 1500..333ms

if (dt >= 333 && dt <= 1500) {

bpmInstant = 60000.0f / (float)dt;

if (bpmInstant >= BPM_MIN && bpmInstant <= BPM_MAX) {

bpmSmooth = pushAvg(bpmBuf, BPM_AVG_N, bpmPtr, bpmInstant);

}

}

}

lastBeatMs = now;

}

// ---------------- SpO2 window fill ----------------

redBuf[spo2Idx] = red;

irBuf[spo2Idx]  = ir;

spo2Idx++;

if (spo2Idx >= N_SPO2) {

spo2Idx = 0;

float s = calcSpO2(redBuf, irBuf, N_SPO2);

if (s > 0) {

// optional calibration mapping

float sCal = SPO2_A * s + SPO2_B;

// clamp

if (sCal > 100) sCal = 100;

if (sCal < 70)  sCal = 70;

spo2Raw = sCal;

if (spo2Filt == 0) spo2Filt = spo2Raw;

spo2Filt = SPO2_ALPHA * spo2Filt + (1.0f - SPO2_ALPHA) * spo2Raw;

}

}

// ---------------- Respiration (experimental) ----------------

// 1) baseline (DC) filter

// Alpha chosen for 100Hz: baseline moves slowly

const float BASE_ALPHA = 0.01f; // ~1s time constant

if (irBaseline == 0) irBaseline = (float)ir;

irBaseline = (1.0f - BASE_ALPHA) * irBaseline + BASE_ALPHA * (float)ir;

// 2) envelope of AC component

float ac = fabs((float)ir - irBaseline);

// fast envelope smoothing

const float ENV_FAST_ALPHA = 0.10f;

envFast = (1.0f - ENV_FAST_ALPHA) * envFast + ENV_FAST_ALPHA * ac;

// slow envelope for respiration (very low frequency)

const float ENV_SLOW_ALPHA = 0.01f;

envSlow = (1.0f - ENV_SLOW_ALPHA) * envSlow + ENV_SLOW_ALPHA * envFast;

// maintain window min/max (for adaptive threshold)

if (millis() - envWindowStart > 15000) { // 15s window

envWindowStart = millis();

envMin = envSlow;

envMax = envSlow;

}

if (envSlow < envMin) envMin = envSlow;

if (envSlow > envMax) envMax = envSlow;

// Peak detect on envSlow using 3-point method

prev2 = prev1;

prev1 = curr;

curr  = envSlow;

// dynamic threshold: mid + 30% of amplitude

float amp = (envMax - envMin);

float thr = envMin + 0.65f * amp;

// detect local max

if (prev2 < prev1 && prev1 > curr && prev1 > thr) {

uint32_t now = millis();

if (lastBreathMs != 0) {

uint32_t dt = now - lastBreathMs;

if (dt >= RR_MIN_MS && dt <= RR_MAX_MS) {

float rrInst = 60000.0f / (float)dt;

rrValue = pushAvg(rrBuf, RR_AVG_N, rrPtr, rrInst);

}

}

lastBreathMs = now;

}

// ---------------- Print once per second ----------------

if (millis() - lastPrintMs >= 1000) {

lastPrintMs = millis();

Serial.print("BPM: ");

if (bpmSmooth > 0) Serial.print(bpmSmooth, 1);

else Serial.print("--");

Serial.print(" | SpO2: ");

if (spo2Filt > 0) Serial.print(spo2Filt, 1);

else Serial.print("--");

Serial.print(" %");

Serial.print(" | RR: ");

if (rrValue > 0) Serial.print(rrValue, 1);

else Serial.print("--");

Serial.println(" bpm");

}

}

}

Pulse sensor

const int pulsePin = 34;

int signalValue;

int threshold = 2000;

int rate[10];

byte rateSpot = 0;

long lastBeat = 0;

float BPM;

int IBI = 600;

bool pulseDetected = false;

int samples[10];

int sampleIndex = 0;

void setup() {

Serial.begin(115200);

Serial.println("Pulse Sensor Ready");

}

void loop() {

signalValue = analogRead(pulsePin);

// ---- Moving Average Filter ----

samples[sampleIndex] = signalValue;

sampleIndex = (sampleIndex + 1) % 10;

int smoothSignal = 0;

for(int i=0;i<10;i++){

smoothSignal += samples[i];

}

smoothSignal /= 10;

// ---- Adaptive Threshold ----

threshold = threshold * 0.9 + smoothSignal * 0.1;

// ---- Beat Detection ----

if(smoothSignal > threshold && pulseDetected == false){

pulseDetected = true;

long currentTime = millis();

IBI = currentTime - lastBeat;

lastBeat = currentTime;

if(IBI > 300){

BPM = 60000.0 / IBI;

Serial.print("BPM:");

Serial.print(BPM);

Serial.print("  Signal:");

Serial.println(smoothSignal);

}

}

if(smoothSignal < threshold){

pulseDetected = false;

}

delay(5);

}

Height sensor calibration code

#include <Wire.h>

#include "Adafruit_VL53L0X.h"

#define SDA_PIN 21

#define SCL_PIN 22

Adafruit_VL53L0X lox;

// ---------- Calibration settings ----------

const int CAL_POINTS = 3;             // You want 3 times

const int CAPTURE_SAMPLES = 15;        // how many samples to average per calibration point

const int RUNTIME_AVG = 3;             // take 2–3 readings before printing calibrated data (set to 3)

// store calibration pairs: rawAvg -> true

float cal_raw[CAL_POINTS];

float cal_true[CAL_POINTS];

// calibration model: true = a*raw + b

float CAL_A = 1.0f;

float CAL_B = 0.0f;

bool calibrated = false;

// ---------- Helpers ----------

void flushSerial() {

while (Serial.available()) Serial.read();

}

void waitForEnter() {

flushSerial();

while (!Serial.available()) delay(10);

// consume until newline

Serial.readStringUntil('\n');

}

int readIntLine() {

while (!Serial.available()) delay(10);

String s = Serial.readStringUntil('\n');

s.trim();

return s.toInt();

}

bool readDistanceOnce(uint16_t &distMM) {

VL53L0X_RangingMeasurementData_t measure;

lox.rangingTest(&measure, false);

if (measure.RangeStatus == 4) return false; // out of range

distMM = measure.RangeMilliMeter;

return true;

}

// average valid readings (skips out-of-range)

bool readDistanceAverage(int samples, float &avgMM) {

long sum = 0;

int good = 0;

// collect until we get "samples" valid reads (with a safety limit)

int attempts = 0;

while (good < samples && attempts < samples * 5) {

attempts++;

uint16_t d;

if (readDistanceOnce(d)) {

sum += d;

good++;

}

delay(30);

}

if (good < (samples / 2)) return false; // too many invalid reads

avgMM = (float)sum / (float)good;

return true;

}

// linear regression: y = a*x + b (best fit)

void computeCalibrationModel() {

float sumx = 0, sumy = 0, sumxx = 0, sumxy = 0;

for (int i = 0; i < CAL_POINTS; i++) {

float x = cal_raw[i];

float y = cal_true[i];

sumx += x;

sumy += y;

sumxx += x * x;

sumxy += x * y;

}

float n = (float)CAL_POINTS;

float denom = (n * sumxx - sumx * sumx);

if (fabs(denom) < 1e-6) {

// fallback: offset-only using first point

CAL_A = 1.0f;

CAL_B = cal_true[0] - cal_raw[0];

} else {

CAL_A = (n * sumxy - sumx * sumy) / denom;

CAL_B = (sumy - CAL_A * sumx) / n;

}

calibrated = true;

}

// run full 3-point calibration

void runCalibration3Point() {

Serial.println("\n==================================");

Serial.println(" VL53L0X 3-POINT CALIBRATION");

Serial.println("==================================");

Serial.println("You will do 3 measurements.");

Serial.println("For each measurement:");

Serial.println("  1) Enter KNOWN distance in mm (example: 150 for 15 cm)");

Serial.println("  2) Place sensor at that distance to the ground/target");

Serial.println("  3) Press ENTER to capture readings\n");

Serial.println("Tip: Use 3 different distances (example: 50mm, 100mm, 150mm) for best accuracy.");

Serial.println("Avoid shiny surfaces. Keep sensor straight/perpendicular.");

for (int i = 0; i < CAL_POINTS; i++) {

Serial.print("\nPoint ");

Serial.print(i + 1);

Serial.print("/");

Serial.print(CAL_POINTS);

Serial.println(":");

Serial.print("Enter KNOWN distance (mm): ");

flushSerial();

int trueMM = readIntLine();

if (trueMM <= 0) {

Serial.println("❌ Invalid input. Try again.");

i--;

continue;

}

Serial.print("Now place sensor at ");

Serial.print(trueMM);

Serial.println(" mm from ground/target. Press ENTER when ready...");

waitForEnter();

float rawAvg = 0;

Serial.println("Capturing... keep still.");

bool ok = readDistanceAverage(CAPTURE_SAMPLES, rawAvg);

if (!ok) {

Serial.println("❌ Too many invalid reads. Re-do this point.");

i--;

continue;

}

cal_true[i] = (float)trueMM;

cal_raw[i]  = rawAvg;

Serial.print("✅ Captured raw average = ");

Serial.print(rawAvg, 2);

Serial.println(" mm");

}

computeCalibrationModel();

Serial.println("\n✅ Calibration finished.");

Serial.println("Calibration points (True -> RawAvg):");

for (int i = 0; i < CAL_POINTS; i++) {

Serial.print("  ");

Serial.print(cal_true[i], 1);

Serial.print(" mm  ->  ");

Serial.print(cal_raw[i], 2);

Serial.println(" mm");

}

Serial.println("\nModel computed:");

Serial.print("  True = A * Raw + B\n  A = ");

Serial.print(CAL_A, 6);

Serial.print("\n  B = ");

Serial.print(CAL_B, 3);

Serial.println("\n");

Serial.println("Now running live calibrated distance.");

Serial.println("Commands:");

Serial.println("  CAL   -> redo 3-point calibration");

Serial.println("  ?     -> show calibration model");

Serial.println();

}

// apply calibration

float applyCal(float rawMM) {

if (!calibrated) return rawMM;

return (CAL_A * rawMM) + CAL_B;

}

void showStatus() {

Serial.println("\n--- STATUS ---");

Serial.print("calibrated = "); Serial.println(calibrated ? "YES" : "NO");

Serial.print("A = "); Serial.println(CAL_A, 6);

Serial.print("B = "); Serial.println(CAL_B, 3);

Serial.println("Points (True -> RawAvg):");

for (int i = 0; i < CAL_POINTS; i++) {

Serial.print("  ");

Serial.print(cal_true[i], 1);

Serial.print(" -> ");

Serial.println(cal_raw[i], 2);

}

Serial.println("-------------\n");

}

void handleCommands() {

if (!Serial.available()) return;

String s = Serial.readStringUntil('\n');

s.trim();

if (s.length() == 0) return;

if (s.equalsIgnoreCase("CAL")) {

runCalibration3Point();

return;

}

if (s == "?") {

showStatus();

return;

}

Serial.println("Unknown command. Use CAL or ? ");

}

void setup() {

Serial.begin(115200);

delay(1200);

Wire.begin(SDA_PIN, SCL_PIN);

Wire.setClock(400000);

Serial.println("\n=== ESP32 + VL53L0X (3-Point Calibration) ===");

Serial.println("Initializing VL53L0X...");

if (!lox.begin()) {

Serial.println("❌ Failed to boot VL53L0X. Check wiring/power.");

while (1) delay(1000);

}

// Stability: longer timing budget

lox.setMeasurementTimingBudgetMicroSeconds(33000);

Serial.println("✅ VL53L0X ready.");

runCalibration3Point();

}

void loop() {

handleCommands();

// live reading: take 2–3 readings before printing calibrated data

float rawAvg = 0;

bool ok = readDistanceAverage(RUNTIME_AVG, rawAvg);

if (!ok) {

Serial.println("Out of range / unstable");

delay(200);

return;

}

float calMM = applyCal(rawAvg);

if (calMM < 0) calMM = 0;

Serial.print("RawAvg(");

Serial.print(RUNTIME_AVG);

Serial.print("): ");

Serial.print(rawAvg, 2);

Serial.print(" mm | Calibrated: ");

Serial.print(calMM, 2);

Serial.print(" mm (");

Serial.print(calMM / 10.0, 1);

Serial.println(" cm)");

delay(300);

}

Combine code with procedure

1) ESP32 “ALL SENSORS + UART to Pi” (NO calibration)

Wiring summary

I2C sensors (MLX90614 + MAX30102 + VL53L0X):

- SDA → ESP32 GPIO 21

- SCL → ESP32 GPIO 22

- VCC → 3.3V

- GND → GND

UART ESP32 ↔ Pi (as you already used):

- ESP32 RX2 GPIO16 ← Pi TX (Pin 8 / GPIO14)

- ESP32 TX2 GPIO17 → Pi RX (Pin 10 / GPIO15)

- GND ↔ GND

HX711

- DT → ESP32 GPIO33

- SCK → ESP32 GPIO25

- VCC → 3.3V

- GND → GND

✅ This avoids pin conflict (UART uses 16/17, HX711 uses 33/25)

✅ Upload this code to ESP32 (Arduino IDE)

Libraries you must install (Arduino Library Manager):

- SparkFun MAX3010x (MAX30105)

- SparkFun Heart Rate (heartRate.h comes with their examples)

- Adafruit VL53L0X

- DFRobot_MLX90614

- HX711 (Rob Tillaart)

/*
  ESP32 ALL-SENSORS + UART to Raspberry Pi
  ---------------------------------------
  Sensors:
    - MLX90614 (Ambient + Object Temp)   [I2C]
    - MAX30102 (BPM + SpO2 + approx RR)  [I2C]
    - VL53L0X (Distance/Height)          [I2C]
    - HX711 (Load cell RAW only)         [GPIO]
  Output:
    - Prints line to Serial Monitor
    - Sends SAME line to Raspberry Pi via UART (Serial2)
  NOTE:
    SpO2 + Respiration are APPROX/DEMO without calibration.
*/

#include <Wire.h>
#include <HardwareSerial.h>

// ---- MLX90614 ----
#include <DFRobot_MLX90614.h>
DFRobot_MLX90614_I2C mlx;

// ---- VL53L0X ----
#include "Adafruit_VL53L0X.h"
Adafruit_VL53L0X lox;

// ---- MAX30102 ----
#include "MAX30105.h"
#include "heartRate.h"
MAX30105 maxSensor;

// ---- HX711 ----
#include "HX711.h"
HX711 hx;

// ===================== PINS =====================
#define I2C_SDA 21
#define I2C_SCL 22

// UART to Raspberry Pi
#define PI_RX 16  // ESP32 RX2  <- Pi TX
#define PI_TX 17  // ESP32 TX2  -> Pi RX
HardwareSerial PiSerial(2);

// HX711 pins (avoid 16/17)
#define HX_DOUT 33
#define HX_SCK  25

// Optional height calculation:
// If you mount VL53L0X at fixed height from floor (mm), set this.
// height_mm = mount_height_mm - distance_mm
#define MOUNT_HEIGHT_MM 0   // 0 = disabled

// ===================== CONFIG =====================
static const uint32_t SEND_EVERY_MS = 1000;
static const uint32_t SENSOR_EVERY_MS = 250;

// Finger detect threshold for MAX30102 IR
#define FINGER_ON_IR 50000

// SpO2 window size
#define N_SPO2 100

// Respiration (approx) smoothing
const uint32_t RR_MIN_MS = 1500;
const uint32_t RR_MAX_MS = 10000;

// ===================== STATE =====================
bool hasMLX = false;
bool hasTOF = false;
bool hasMAX = false;
bool hasHX  = false;

float ambC = 0, objC = 0;

int tofMM = -1;
int heightMM = -1;

long hxRaw = 0;

// MAX30102 values
float bpmSmooth = 0;
float spo2Filt = 0;
float rrValue = 0;

// SpO2 buffers
uint32_t redBuf[N_SPO2];
uint32_t irBuf[N_SPO2];
int spo2Idx = 0;

// BPM
uint32_t lastBeatMs = 0;
float bpmBuf[6] = {0};
int bpmPtr = 0;

// Resp estimation from envelope
float irBaseline = 0;
float envFast = 0;
float envSlow = 0;
float envMin = 1e9, envMax = 0;
uint32_t envWindowStart = 0;
float prev2 = 0, prev1 = 0, curr = 0;
uint32_t lastBreathMs = 0;
float rrBuf[5] = {0};
int rrPtr = 0;

uint32_t lastSendMs = 0;
uint32_t lastSensorMs = 0;

// ===================== HELPERS =====================
void scanI2C() {
  Serial.println("\nI2C scan:");
  int cnt = 0;
  for (uint8_t addr = 1; addr < 127; addr++) {
    Wire.beginTransmission(addr);
    uint8_t e = Wire.endTransmission();
    if (e == 0) {
      Serial.print("  Found 0x");
      if (addr < 16) Serial.print("0");
      Serial.println(addr, HEX);
      cnt++;
    }
  }
  Serial.print("Total I2C devices: ");
  Serial.println(cnt);
}

float mean_u32(const uint32_t *x, int n){
  double s=0;
  for(int i=0;i<n;i++) s += x[i];
  return (float)(s/n);
}
float rms_ac(const uint32_t *x, int n, float dc){
  double s=0;
  for(int i=0;i<n;i++){
    double d = (double)x[i] - dc;
    s += d*d;
  }
  return (float)sqrt(s/n);
}
float calcSpO2(const uint32_t *red, const uint32_t *ir, int n){
  float dcRed = mean_u32(red,n);
  float dcIr  = mean_u32(ir,n);
  if(dcRed<=0 || dcIr<=0) return -1;

  float acRed = rms_ac(red,n,dcRed);
  float acIr  = rms_ac(ir,n,dcIr);
  if(acRed<=0 || acIr<=0) return -1;

  float R = (acRed/dcRed)/(acIr/dcIr);

  // demo approximation (needs real calibration for medical accuracy)
  float s = 110.0f - 25.0f * R;
  if(s>100) s=100;
  if(s<70)  s=70;
  return s;
}

float pushAvg(float *buf, int n, int &ptr, float v){
  buf[ptr] = v;
  ptr = (ptr + 1) % n;
  float sum = 0; int cnt = 0;
  for(int i=0;i<n;i++){
    if(buf[i] > 0){
      sum += buf[i];
      cnt++;
    }
  }
  return (cnt>0) ? (sum/cnt) : 0;
}

void resetMAX() {
  spo2Idx = 0;
  spo2Filt = 0;

  bpmSmooth = 0;
  lastBeatMs = 0;
  for(int i=0;i<6;i++) bpmBuf[i]=0;
  bpmPtr = 0;

  rrValue = 0;
  for(int i=0;i<5;i++) rrBuf[i]=0;
  rrPtr = 0;

  irBaseline = 0;
  envFast = 0;
  envSlow = 0;
  envMin = 1e9;
  envMax = 0;
  envWindowStart = millis();
  prev2 = prev1 = curr = 0;
  lastBreathMs = 0;
}

// ===================== INIT =====================
void setup() {
  Serial.begin(115200);
  delay(1200);

  Serial.println("\n=== ESP32 ALL SENSORS + UART ===");

  // UART to Raspberry Pi
  PiSerial.begin(115200, SERIAL_8N1, PI_RX, PI_TX);
  Serial.println("UART2 ready (GPIO16/17) -> Raspberry Pi");

  // I2C
  Wire.begin(I2C_SDA, I2C_SCL);
  Wire.setClock(100000); // safe start for multiple sensors
  scanI2C();

  // MLX90614
  if (mlx.begin() == NO_ERR) {
    hasMLX = true;
    Serial.println("✅ MLX90614 detected");
  } else {
    Serial.println("⚠️ MLX90614 not detected");
  }

  // VL53L0X
  if (lox.begin()) {
    hasTOF = true;
    lox.setMeasurementTimingBudgetMicroSeconds(33000);
    Serial.println("✅ VL53L0X detected");
  } else {
    Serial.println("⚠️ VL53L0X not detected");
  }

  // MAX30102
  if (maxSensor.begin(Wire, I2C_SPEED_STANDARD)) {
    hasMAX = true;
    Serial.println("✅ MAX30102 detected");

    byte ledBrightness = 0x1F;
    byte sampleAverage = 4;
    byte ledMode = 2;
    int sampleRate = 100;
    int pulseWidth = 411;
    int adcRange = 4096;

    maxSensor.setup(ledBrightness, sampleAverage, ledMode, sampleRate, pulseWidth, adcRange);
    maxSensor.setPulseAmplitudeRed(0x1F);
    maxSensor.setPulseAmplitudeIR(0x1F);
    maxSensor.setPulseAmplitudeGreen(0);
    maxSensor.clearFIFO();

    resetMAX();
  } else {
    Serial.println("⚠️ MAX30102 not detected");
  }

  // HX711
  hx.begin(HX_DOUT, HX_SCK);
  pinMode(HX_DOUT, INPUT);
  // if DOUT never goes low, it will not be ready
  hasHX = true; // we'll still try
  Serial.println("✅ HX711 configured (RAW only)");

  Serial.println("\nOutput format: CSV line -> Serial + Pi UART");
  Serial.println("ts,ambC,objC,bpm,spo2,rr,distMM,heightMM,hxRaw");
}

// ===================== SENSOR READS =====================
void readMLX() {
  if (!hasMLX) return;
  ambC = mlx.getAmbientTempCelsius();
  objC = mlx.getObjectTempCelsius();
}

void readTOF() {
  if (!hasTOF) return;
  VL53L0X_RangingMeasurementData_t m;
  lox.rangingTest(&m, false);
  if (m.RangeStatus == 4) {
    tofMM = -1;
    heightMM = -1;
    return;
  }
  tofMM = (int)m.RangeMilliMeter;
  if (MOUNT_HEIGHT_MM > 0) {
    heightMM = MOUNT_HEIGHT_MM - tofMM;
    if (heightMM < 0) heightMM = 0;
  } else {
    heightMM = -1;
  }
}

void readHX() {
  // RAW only (no scale/offset)
  if (!hasHX) return;

  // wait up to 100ms
  if (!hx.wait_ready_timeout(100)) {
    // not ready
    return;
  }

  // average a few raw reads
  long sum = 0;
  const int N = 8;
  for (int i = 0; i < N; i++) {
    sum += hx.read();
    delay(5);
  }
  hxRaw = sum / N;
}

void processMAX() {
  if (!hasMAX) return;

  maxSensor.check();
  while (maxSensor.available()) {
    uint32_t red = maxSensor.getFIFORed();
    uint32_t ir  = maxSensor.getFIFOIR();
    maxSensor.nextSample();

    // finger detect
    if (ir < FINGER_ON_IR) {
      resetMAX();
      return;
    }

    // BPM
    if (checkForBeat(ir)) {
      uint32_t now = millis();
      if (lastBeatMs != 0) {
        uint32_t dt = now - lastBeatMs;
        if (dt >= 333 && dt <= 1500) {
          float bpm = 60000.0f / (float)dt;
          bpmSmooth = pushAvg(bpmBuf, 6, bpmPtr, bpm);
        }
      }
      lastBeatMs = now;
    }

    // SpO2 window
    redBuf[spo2Idx] = red;
    irBuf[spo2Idx]  = ir;
    spo2Idx++;

    if (spo2Idx >= N_SPO2) {
      spo2Idx = 0;
      float s = calcSpO2(redBuf, irBuf, N_SPO2);
      if (s > 0) {
        if (spo2Filt == 0) spo2Filt = s;
        spo2Filt = 0.7f * spo2Filt + 0.3f * s;
      }
    }

    // Respiration (approx from envelope)
    const float BASE_ALPHA = 0.01f;
    if (irBaseline == 0) irBaseline = (float)ir;
    irBaseline = (1.0f - BASE_ALPHA) * irBaseline + BASE_ALPHA * (float)ir;

    float ac = fabs((float)ir - irBaseline);

    const float ENV_FAST_ALPHA = 0.10f;
    envFast = (1.0f - ENV_FAST_ALPHA) * envFast + ENV_FAST_ALPHA * ac;

    const float ENV_SLOW_ALPHA = 0.01f;
    envSlow = (1.0f - ENV_SLOW_ALPHA) * envSlow + ENV_SLOW_ALPHA * envFast;

    if (millis() - envWindowStart > 15000) {
      envWindowStart = millis();
      envMin = envSlow;
      envMax = envSlow;
    }
    if (envSlow < envMin) envMin = envSlow;
    if (envSlow > envMax) envMax = envSlow;

    prev2 = prev1;
    prev1 = curr;
    curr  = envSlow;

    float amp = (envMax - envMin);
    float thr = envMin + 0.65f * amp;

    if (prev2 < prev1 && prev1 > curr && prev1 > thr) {
      uint32_t now = millis();
      if (lastBreathMs != 0) {
        uint32_t dt = now - lastBreathMs;
        if (dt >= RR_MIN_MS && dt <= RR_MAX_MS) {
          float rr = 60000.0f / (float)dt;
          rrValue = pushAvg(rrBuf, 5, rrPtr, rr);
        }
      }
      lastBreathMs = now;
    }
  }
}

// ===================== UART COMMANDS (optional) =====================
void handlePiCommands() {
  // If Pi sends "PING\n" you reply "ESP_OK\n"
  static String buf;
  while (PiSerial.available()) {
    char c = (char)PiSerial.read();
    if (c == '\n' || c == '\r') {
      buf.trim();
      if (buf.length()) {
        if (buf.equalsIgnoreCase("PING")) {
          PiSerial.println("ESP_OK");
        } else if (buf.equalsIgnoreCase("ID")) {
          PiSerial.println("ESP32_HEALTH_STATION");
        } else {
          PiSerial.print("ESP_ACK:");
          PiSerial.println(buf);
        }
      }
      buf = "";
    } else {
      if (buf.length() < 120) buf += c;
    }
  }
}

// ===================== SEND LINE =====================
void sendLine() {
  // CSV (easy to read in Pi terminal)
  // ts,ambC,objC,bpm,spo2,rr,distMM,heightMM,hxRaw
  char line[220];
  snprintf(line, sizeof(line),
           "%lu,%.2f,%.2f,%.1f,%.1f,%.1f,%d,%d,%ld",
           (unsigned long)millis(),
           ambC, objC,
           bpmSmooth,
           spo2Filt,
           rrValue,
           tofMM, heightMM,
           hxRaw);

  Serial.println(line);
  PiSerial.println(line);
}

// ===================== LOOP =====================
void loop() {
  handlePiCommands();

  // Always process MAX FIFO (fast)
  processMAX();

  // Periodic reads for other sensors
  if (millis() - lastSensorMs >= SENSOR_EVERY_MS) {
    lastSensorMs = millis();
    readMLX();
    readTOF();
    readHX();
  }

  // Send to Pi + Serial every 1 second
  if (millis() - lastSendMs >= SEND_EVERY_MS) {
    lastSendMs = millis();
    sendLine();
  }

  delay(5);
}

2) Raspberry Pi: read and print in terminal (best way)

Install pyserial (one time)

pip3 install pyserial

Create file read_uart.py

import serial

PORT = "/dev/serial0"
BAUD = 115200

ser = serial.Serial(PORT, BAUD, timeout=1)

print("Listening on", PORT, "at", BAUD)
print("Press Ctrl+C to stop\n")

try:
    while True:
        line = ser.readline().decode(errors="replace").strip()
        if line:
            print(line, flush=True)
except KeyboardInterrupt:
    pass
finally:
    ser.close()

Run:

python3 -u read_uart.py

✅ You should see lines like:

12000,24.32,33.18,72.5,97.1,15.2,120, -1, 987654

Notes (important, so you don’t get “garbage characters”)

- Always use 115200 on both sides

- Pi UART must be enabled (you already did)

- You must have common GND

- If you see “weird symbols” on Pi: run:

sudo stty -F /dev/serial0 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts -icanon -echo

What you’ll do next (after this works)